This document provides detailed specifications for the GGEL Series heaters. It includes power ratings, voltage, recommended mounting heights, coverage areas, and key notes.

04760240 – 6000 Watt Heater

- • Power Rating: 6.0kW (6000W)

- • Voltage: 240V (Approximately derated to 4.5 kW at 208V)

- • Recommended Mounting Height: Minimum 8 ft (2.4 m) from floor

- • Minimum Clearances: Ceiling 12 in (304 mm), Ends 18 in (457 mm), Sides 24 in (609 mm)

- • Coverage Area: Approximately 120–150 sq ft depending on exposure and installation

- 04730240 & 04730240D – 3000 Watt Heaters

- • Power Rating: 3.0 kW (3000W)

- • Voltage: 240V (Approximately derated to 2.25 kW at 208V)

- • Recommended Mounting Height: Minimum 8 ft (2.4 m) from floor

- • Minimum Clearances: Ceiling 12 in (304 mm), Ends 18 in (457 mm), Sides 24 in (609 mm)

- • Coverage Area: Approximately 80–100 sq ft depending on exposure and installation

- 04715120 – 1500 Watt Heater

- • Power Rating: 1.5 kW (1500W)

- • Voltage: 120V

- • Recommended Mounting Height: Minimum 6 ft 7 in (2.0 m) from floor

- • Minimum Clearances: Ceiling 12 in (304 mm), Ends 18 in (457 mm)

- • Coverage Area: Approximately 40–60 sq ft depending on exposure and installation

All models require 6 ft frontal clearance to combustibles. Maintain proper clearances installation per the manual.

Emitter Technology

- Carbon fiber braid emitters ensure high-performance, medium-wave infrared heating with minimal visible glow and typical emitter service up to 8000 hours under normal operating conditions.

Housing & Frame

- All models utilize an extruded aircraft-grade aluminum frame. All models feature a powder-coated aluminum housing designed for durability and reduced surface temperatures

Mounting Components

- Wall-mount brackets included for all models.

- Ceiling-suspension rods/brackets (12″ length) included for 3000W and higher models.

Environmental Protection & Certification

- IP65 rated — dust-tight and protected against low pressure water jets from any direction.

- Certified: (cETLus / UL2021 / CSA22.2)

Controls & Electronics

- Onboard control switch paired with a handheld remote featuring multi-level heat settings (2-stage for 1500W/3000W, 6-stage for 3000W).

Additional Features

- Low-maintenance, silent infrared heating with no odor or emissions, reaching full output in 3 seconds.